Pcdmistoexcel_auto

To install on a new computer:

1)  copy all files to c drive

2) from the start, run , open line

type  regsvr32 c:\pcdmisutils.dll

Note: there is a space between the 2 and the c in the line above

3) set the security level in excel to medium( found under tool, macro, security)

4) To automatically send results to excel, place this command at the end of your pcdmis program:

CS1        =SCRIPT/FILENAME= C:\EXCELSCRIPT.BAS

FUNCTION/LoadExcel,,

STARTSCRIPT/

ENDSCRIPT/

5) Save the excel results file as a different name