5-1-06

DataPage start procedure

A database folder must be created in the DataPage DP Editor before PC-DMIS can use it.

The DP Editor can be opened from the Start menu.

NOTE: Once opened, the title bar at the top of the window will be “DataPage/RT Editor”

The DataPage Editor can also be opened from the main DataPage program (File, Editor).

Select “Database, New…”.

The “Create New Database” window will open.  A full path name for the new folder can be keyed in.

The easier method is to click on “Browse” and select “DATAPAGE” from the “c:\” folder.  Click “OK”.

The result will look like this.

Key in a backslash (\) and create a name for the new database folder with 8 or less characters, with letters in uppercase.  Click “OK”.

If the new database folder name does not exist, clicking “Yes” in the new window will create it.

If the new database folder name already exists, an option to overwrite it will be given.

The new database folder can now be selected in PC-DMIS.

Before measurements are made, insert “Statistics”.

In the “Statistics Options” window, make sure that “Database Options” has “DataPage” selected.

Click on “Use Dimension Name” if there are any distances between features that must be saved, because these do not have feature names.

Click the “Add Directory To List…” button.

A newly created database folder will not be in the list.  Click on the button next to “Statistics Directory to Add:”.  In the “Choose Directory” window, go to “C:\Datapage” and the click on the database folder name.  Click “OK”.

In the “Add Directory To List” window,  Click “OK”.

In the “Statistics Options” window,  Click “OK”.

The resulting programming lines will look like this:

STATS/DATABASE,DATAPAGE,$

DIRECTORY=C:\DATAPAGE\PART_123,$

DIRECTORY=,$

READ=40,WRITE=40,MEMPAGES=64,DIMENSION NAME,CONTROLCALC ON,$

STATS/END

When executing the PC-DMIS program, a “Save Statistics” window will open at the programmed command with the current statistics count.  Click “Yes” to save.

At the end of the executed program, an “Update” window will open.  Click “Yes” to save.